This report explains the implementation of an Access Control List (ACL) on the Layer 3 switch within Autonomous Systems : AS100 , AS200 , AS300 and AS400.
The purpose of the ACL is to restrict unauthorized communication between hosts in the local network to enhance overall security.

Blocking PC14 (IP: 192.168.1.5) from accessing the Server (IP: 192.168.1.100).

Allowing all other devices to communicate normally with the server

Blocking PC24 (IP: 192.168.2.5) from accessing the Server (IP: 192.168.2.100).

Allowing all other devices to communicate normally with the server

Blocking PC14 (IP: 192.168.3.5) from accessing the Server (IP: 192.168.3.100).

Allowing all other devices to communicate normally with the server

Blocking PC14 (IP: 192.168.4.5) from accessing the Server (IP: 192.168.4.100).

Allowing all other devices to communicate normally with the server

- Configuration

- AS100

<LW1> system-view

# -------- ACL Configuration --------

[LW1] acl number 3000

[LW1-acl-3000] rule 5 deny ip source 192.168.1.5 0.0.0.0 destination 192.168.1.100 0.0.0.0

[LW1-acl-3000] rule 10 permit ip

[LW1-acl-3000] quit

# -------- Apply ACL on Vlan Gateway --------

[LW1] interface Vlanif10

[LW1-Vlanif10] traffic-filter inbound acl 3000

[LW1-Vlanif10] quit

<LW1> save

- AS200

<LSW4> system-view

[LSW4] acl number 3001

[LSW4-acl-3001] rule 5 deny ip source 192.168.2.5 0.0.0.0 destination 192.168.2.100 0.0.0.0

[LSW4-acl-3001] rule 10 permit ip

[LSW4-acl-3001] quit

[LSW4] interface Vlanif30

[LSW4-Vlanif30] traffic-filter inbound acl 3001

[LSW4-Vlanif30] quit

< LSW4> save

- AS300

<LSW14> system-view

[LSW14] acl number 3002

[LSW14-acl-3002] rule 5 deny ip source 192.168.3.5 0.0.0.0 destination 192.168.3.100 0.0.0.0

[LSW14-acl-3002] rule 10 permit ip

[LSW14-acl-3002] quit

[LSW14] interface Vlanif50

[LSW14-Vlanif50] traffic-filter inbound acl 3002

[LSW14-Vlanif50] quit

< LSW14> save

- AS400

<LSW17> system-view

[LSW17] acl number 3003

[LSW17-acl-3003] rule 5 deny ip source 192.168.4.5 0.0.0.0 destination 192.168.4.100 0.0.0.0

[LSW17-acl-3003] rule 10 permit ip

[LSW17-acl-3003] quit

[LSW17] interface Vlanif70

[LSW17-Vlanif70] traffic-filter inbound acl 3003

[LSW17-Vlanif70] quit

< LSW17> save

- Test

AS100

AS200

AS300

AS400